# Group Member

Group name : ต้นน้ำทรงพลัง

| Name | Student Number |
| --- | --- |
| Chanasuek Chaiyasorn | 6731309021 |
| Saranpat Pewsouykam | 6730489021 |
| Sarus Suanploy | 6730493521 |

# Overall Design Block Diagram

Real-time 640×480 video pipeline on Basys 3 + OV7670. Pixels stream from the camera at 24 MHz, get quantized to YCbCr 4:2:2 (Y3 + Cb2/Cr2) for BRAM storage, and are read back at 25 MHz for VGA output through a filter pipeline.

clk_100 (Basys3 100 MHz crystal) ¦ v clk_wiz_main (MMCM) ──── clk_25 ──── VGA pixel + main logic ──── clk_24 ──── cam XCLK → OV7670 OV7670 sensor ¦ RGB565 (2 bytes/pixel) over D[7:0], PCLK, HREF, VSYNC v cam_capture RGB565 → YCbCr 4:2:2 (Y3, Cb2, Cr2) ¦ write @ cam_pclk (~24 MHz) v frame_buffer split BRAM: luma_mem 307,200 × 3 b (~30 BRAM36) chroma_mem 153,600 × 4 b (~20 BRAM36) ¦ read @ clk_25 v filter_pipeline SW[1:0] selects mode: 00 = RAW (decoded YCbCr → RGB444) 01 = INVERT (bitwise NOT of RGB444) 10 = COLOR_ISO (gate channels by SW[3:2]) 11 = EDGE (3×3 Sobel on Y3, threshold SW[7:4]) ¦ (EDGE mode uses line_buffer3 + filter_edge) v ycbcr_to_rgb444 Y3 + dither → Y4; LUT offsets → clamp → 12-bit RGB444 ¦ v vga_timing ──► HSYNC / VSYNC / RGB444 to VGA connector ─ Boot path ────────────────────────────────────────────────────────── cam_config (77-reg SCCB ROM) → sccb_master → SIOC/SIOD → OV7670

# Demo Video

Demo recording (Google Drive):

[Insert Google Drive / YouTube link here]

# Summary - Filter Selection

Per the project rubric (3 distinct hardware-based filters, togglable in real-time via Basys 3 slide switches), this group implemented the following:

Color Inversion (Negative) — bitwise NOT of the decoded RGB444 output. Selected via SW[1:0] = 2'b01.

Color Channel Isolation — pass-through of a single R / G / B channel chosen by SW[3:2]. Selected via SW[1:0] = 2'b10.

Sobel Edge Detection — 3x3 convolution on luma (Y3) using a sliding line buffer, with threshold tunable via SW[7:4]. Selected via SW[1:0] = 2'b11.

SW[1:0] = 2'b00 displays the raw decoded video stream (YCbCr 4:2:2 -> RGB444 with 2x2 Bayer dither).

# Design Decision

## 1. Module Architecture

- Why did you structure your design this way?

○ Answer : Each functional block lives in its own file (cam_capture, frame_buffer, filter_pipeline, ycbcr_to_rgb444, sccb_master, vga_timing). Clock-domain boundaries (cam_pclk vs clk_25) are isolated to specific modules so that timing constraints stay simple. The top-level only wires everything together and owns the reset-and-config FSM.

- What are the advantages of using this approach?

○ Answer : Modularity makes it easy to swap implementations (e.g. RGB444 vs YCbCr 4:2:2 storage), debug each stage in isolation with a dedicated testbench, and reason about clock-domain crossings since they are confined to frame_buffer and cdc_pulse_sync.

## 2. Communication Protocols

- Why did you choose this specific protocol for communication?

○ Answer : SCCB (Serial Camera Control Bus, OmniVision-defined, ~95% I2C-compatible) is the only configuration interface OV7670 exposes, so it is mandatory for setting RGB565 output, resolution, gain, and AWB. The pixel data path itself is a synchronous parallel bus (PCLK / HREF / VSYNC / D[7:0]).

- How does this protocol suit your design requirements?

○ Answer : Configuration is one-shot and slow (~100 kHz SCCB), so bit-banging it from clk_25 is cheap and never gets in the way of the high-throughput pixel bus. The parallel bus on the data side gives one byte per PCLK with no protocol overhead, which matches the 25 MHz fill rate VGA needs.

## 3. Clock and Data Transfer Rate

- Why did you select this specific clock frequency?

○ Answer : Two clocks are derived by the MMCM from the 100 MHz board crystal: 25.000 MHz for VGA pixel + main logic (matches VESA 640×480@60 Hz timing) and 24.000 MHz for the camera XCLK (OV7670 datasheet typical). The frame buffer is dual-clock (write @ 24 MHz, read @ 25 MHz) so each domain runs at its native rate.

- How does the data rate impact system performance?

○ Answer : VGA at 25 MHz consumes one pixel per cycle = 60 frames/sec. OV7670 streams ~30 fps at 24 MHz XCLK. Because the frame buffer decouples producer and consumer, brief rate differences are absorbed without tearing. Heavy math (BT.601 luma multiply, Sobel) sits on the read side at 25 MHz, leaving the write side light.

- Did you consider constraints such as FPGA timing, external device compatibility, or noise margins?

○ Answer : Yes. Both 24 MHz and 25 MHz come from the same MMCM VCO (1200 MHz) so they share a known phase relationship; CDC is still treated explicitly via a dual-port BRAM and cdc_pulse_sync for VSYNC. SCCB is held at ~100 kHz to stay well within OV7670 spec and to tolerate breadboard pull-up tolerances.

## 4. Resource Utilization

- How does your design balance resource usage (LUTs, FFs, BRAMs) and performance?

○ Answer : The 640×480 frame would not fit in BRAM at full RGB565 (16 bpp → 4.9 Mb), so the design quantizes to YCbCr 4:2:2 with Y3 + Cb2 + Cr2 = 5 bits/pixel = 1.54 Mb total. Luma uses ~30 BRAM36 tiles, chroma ~20, hitting full utilization on XC7A35T. LUTs are spent on the BT.601 multiply (write side) and the Sobel kernel + threshold (read side). FFs hold pipeline registers and the SCCB FSM.

- Why did you optimize certain parts of the design?

○ Answer : Hot paths are the per-pixel converter (write side) and the Sobel + line buffer (read side). Y is computed combinationally from RGB, then registered before BRAM. The Cb/Cr 4-bin quantizer keeps thresholds antisymmetric (-48, 0, +48) so a neutral pixel round-trips back to neutral. The decoder uses small case-statement LUTs for r_off/b_off/g_off, sums and clamps to RGB444 — no DSPs needed on the read side.

# Implementation Detail

## Module : top

Module code:

`timescale 1ns/1ps // Top-level: real-time 640x480 video pipeline on Basys 3 + OV7670. // // Data path: // OV7670 (RGB565 @ pclk) // -> cam_capture (RGB565 -> YCbCr 4:2:2, Y3+Cb2/Cr2) // -> frame_buffer (split luma 30 BRAMs + chroma 20 BRAMs) // -> filter_pipeline @ clk_25 (RAW / INVERT / COLOR_ISO / EDGE) // -> VGA pins (RGB444) // // Clocks: // clk_100 (board) -> clk_wiz_main -> clk_25 (VGA pixel) and clk_24 (cam XCLK) // // Switch mapping: // SW[1:0] filter mode // SW[3:2] color isolation channel // SW[7:4] Sobel edge threshold // SW[8] OV7670 internal color-bar test pattern (SCCB writes) // SW[9] FPGA-side test pattern bypass (replaces camera writes) // SW[15:10] reserved // // LED map: // LD[15] cfg_done // LD[14] led_frame_toggle (toggles each VSYNC) // LD[13:8] reg_writes_done[5:0] // LD[7:4] sw74 passthrough // LD[3:2] sw32 passthrough // LD[1:0] sw_mode passthrough module top #( parameter integer DEBOUNCE_N = 20, parameter integer CAM_RST_HOLD = 625000, parameter integer CAM_POST_RST = 250000, parameter integer CAM_SOFT_WAIT = 750000, parameter integer CAM_REG_GAP = 2500 ) ( input wire clk_100, output wire vga_hsync, output wire vga_vsync, output wire [3:0] vga_r, output wire [3:0] vga_g, output wire [3:0] vga_b, output wire cam_xclk, output wire cam_rst_n, output wire cam_pwdn, output wire cam_scl, inout wire cam_sda, input wire cam_pclk, input wire cam_href, input wire cam_vsync, input wire [7:0] cam_d, input wire [15:0] sw, input wire btnc, output wire [15:0] led ); // ----------------------------------------------------------------- // Clocks // ----------------------------------------------------------------- wire clk_25, clk_24, mmcm_locked; clk_wiz_main u_clkwiz ( .clk_100 (clk_100), .clk_25 (clk_25), .clk_24 (clk_24), .locked (mmcm_locked) ); assign cam_xclk = clk_24; // ----------------------------------------------------------------- // Debounce switches and center button (clk_25 domain) // ----------------------------------------------------------------- wire [15:0] sw_db; wire btnc_db; debouncer #(.W(16), .N(DEBOUNCE_N)) u_sw_db ( .clk (clk_25), .sw_in (sw), .sw_out (sw_db) ); debouncer #(.W(1), .N(DEBOUNCE_N)) u_btn_db ( .clk (clk_25), .sw_in (btnc), .sw_out (btnc_db) ); wire sys_rst = btnc_db ¦ ~mmcm_locked; wire [1:0] sw_mode = sw_db[1:0]; wire [1:0] sw32 = sw_db[3:2]; wire [3:0] sw74 = sw_db[7:4]; wire sw_test_pattern = sw_db[8]; wire tp_enable = sw_db[9]; // ----------------------------------------------------------------- // Camera configuration (SCCB) // ----------------------------------------------------------------- wire cfg_done; wire [6:0] reg_writes_done; wire [6:0] successful_xacts; cam_config #( .RST_HOLD (CAM_RST_HOLD), .POST_RST (CAM_POST_RST), .SOFT_RST_WAIT (CAM_SOFT_WAIT), .REG_GAP (CAM_REG_GAP), .OV7670_ID (8'h42) ) u_camcfg ( .clk (clk_25), .rst (sys_rst), .cam_rst_n (cam_rst_n), .cam_pwdn (cam_pwdn), .scl (cam_scl), .sda (cam_sda), .test_pattern_en (sw_test_pattern), .cfg_done (cfg_done), .reg_writes_done (reg_writes_done), .successful_xacts (successful_xacts) ); // ----------------------------------------------------------------- // Camera capture (cam_pclk domain) -> YCbCr 4:2:2 // ----------------------------------------------------------------- wire [18:0] cap_addr; wire [2:0] cap_luma; wire [3:0] cap_chroma; wire cap_we; wire cap_chroma_we; wire vsync_pulse_pclk; cam_capture #( .IMG_WIDTH (640), .IMG_HEIGHT (480) ) u_capture ( .pclk_in (cam_pclk), .d_in (cam_d), .href (cam_href), .vsync (cam_vsync), .wr_addr (cap_addr), .wr_luma (cap_luma), .wr_chroma (cap_chroma), .wr_en (cap_we), .wr_chroma_en (cap_chroma_we), .vsync_pulse (vsync_pulse_pclk) ); // ----------------------------------------------------------------- // FPGA-side test pattern (SW[9]) — three vertical color bars in YCbCr // Red bar: Y=4, {Cb2=01 (mild blue deficit), Cr2=11 (strong red)} // Green bar: Y=5, {Cb2=00, Cr2=00} (both deficit) // Blue bar: Y=2, {Cb2=11 (strong blue), Cr2=01 (mild)} // ----------------------------------------------------------------- reg [9:0] tp_col, tp_row; reg [18:0] tp_addr; reg [2:0] tp_luma; reg [3:0] tp_chroma; reg tp_we, tp_chroma_we; always @(posedge cam_pclk) begin if (sys_rst) begin tp_col <= 10'd0; tp_row <= 10'd0; end else if (tp_col == 10'd639) begin tp_col <= 10'd0; tp_row <= (tp_row == 10'd479) ? 10'd0 : tp_row + 10'd1; end else begin tp_col <= tp_col + 10'd1; end end always @(*) begin tp_addr = tp_row * 19'd640 + {9'd0, tp_col}; tp_we = 1'b1; tp_chroma_we = ~tp_col[0]; if (tp_col < 10'd213) begin tp_luma = 3'd5; // Red: strong Cr, strong B deficit tp_chroma = 4'b00_11; // Cb=00 (strong B deficit), Cr=11 (strong R) end else if (tp_col < 10'd426) begin tp_luma = 3'd5; // Green: both strong deficit -> G wins tp_chroma = 4'b00_00; // Cb=00, Cr=00 end else begin tp_luma = 3'd5; // Blue: strong Cb, strong R deficit tp_chroma = 4'b11_00; // Cb=11 (strong B), Cr=00 (strong R deficit) end end wire [18:0] fb_wr_addr = tp_enable ? tp_addr : cap_addr; wire [2:0] fb_wr_luma = tp_enable ? tp_luma : cap_luma; wire [3:0] fb_wr_chroma = tp_enable ? tp_chroma : cap_chroma; wire fb_wr_en = tp_enable ? tp_we : cap_we; wire fb_wr_chroma_en = tp_enable ? tp_chroma_we : cap_chroma_we; // ----------------------------------------------------------------- // Frame buffer (write @ cam_pclk, read @ clk_25) // ----------------------------------------------------------------- wire [18:0] fb_rd_addr; wire [2:0] fb_rd_luma; wire [3:0] fb_rd_chroma; frame_buffer u_fb ( .wr_clk (cam_pclk), .wr_en (fb_wr_en), .wr_addr (fb_wr_addr), .wr_luma (fb_wr_luma), .wr_chroma (fb_wr_chroma), .wr_chroma_en (fb_wr_chroma_en), .rd_clk (clk_25), .rd_addr (fb_rd_addr), .rd_luma (fb_rd_luma), .rd_chroma (fb_rd_chroma) ); // ----------------------------------------------------------------- // VGA timing // ----------------------------------------------------------------- wire [9:0] h_count, v_count; wire active_video; wire fetch_active; vga_timing u_vga ( .clk (clk_25), .rst (sys_rst), .hsync (vga_hsync), .vsync (vga_vsync), .h_count (h_count), .v_count (v_count), .active_video (active_video), .fetch_active (fetch_active), .rd_addr (fb_rd_addr) ); wire h_edge = (h_count == 10'd0) ¦ (h_count >= 10'd638); wire v_edge = (v_count < 10'd2) ¦ (v_count >= 10'd479); // ----------------------------------------------------------------- // Filter pipeline // ----------------------------------------------------------------- wire [11:0] filter_out; filter_pipeline #(.WIDTH(640)) u_filt ( .clk (clk_25), .rst (sys_rst), .fb_luma (fb_rd_luma), .fb_chroma (fb_rd_chroma), .h_count (h_count), .v_count (v_count), .pix_valid (active_video), .sw_mode (sw_mode), .sw32 (sw32), .sw74 (sw74), .h_edge (h_edge), .v_edge (v_edge), .rgb444_out (filter_out) ); // ----------------------------------------------------------------- // VGA output gating // Pipeline depth from h_count to filter_out: // rd_addr comb (0) + frame_buffer BRAM (1) + filter_pipeline (2) = 3 // Delay active_video by 3 cycles to gate output cleanly. // ----------------------------------------------------------------- reg [2:0] active_pipe; always @(posedge clk_25) begin if (sys_rst) active_pipe <= 3'd0; else active_pipe <= {active_pipe[1:0], fetch_active}; end wire active_d = active_pipe[2] && active_video; assign vga_r = active_d ? filter_out[11:8] : 4'h0; assign vga_g = active_d ? filter_out[7:4] : 4'h0; assign vga_b = active_d ? filter_out[3:0] : 4'h0; // ----------------------------------------------------------------- // VSYNC pulse CDC: cam_pclk -> clk_25, drives a frame-rate LED // ----------------------------------------------------------------- wire vsync_pulse_25; cdc_pulse_sync u_cdc ( .clk_src (cam_pclk), .clk_dst (clk_25), .rst_src (1'b0), .rst_dst (sys_rst), .pulse_in (vsync_pulse_pclk), .pulse_out (vsync_pulse_25) ); reg led_frame_toggle; always @(posedge clk_25) begin if (sys_rst) led_frame_toggle <= 1'b0; else if (vsync_pulse_25) led_frame_toggle <= ~led_frame_toggle; end /* verilator lint_off UNUSED */ wire _unused_xacts = &{1'b0, successful_xacts}; /* verilator lint_on UNUSED */ assign led[15] = cfg_done; assign led[14] = led_frame_toggle; assign led[13:8] = reg_writes_done[5:0]; assign led[7:4] = sw74; assign led[3:2] = sw32; assign led[1:0] = sw_mode; endmodule

Module Description:

Top-level wrapper. Instantiates the MMCM (clk_25 + clk_24), debouncer, sccb_master + cam_config, cam_capture, frame_buffer, filter_pipeline (which embeds line_buffer3, filter_edge, and ycbcr_to_rgb444), and vga_timing. Owns the reset hold / camera power-up sequence and routes SW[15:0] into per-module mode and threshold inputs. LD[15] indicates SCCB config done; LD[14] toggles every camera VSYNC; LD[13:8] show reg_writes_done count.

## Module : clk_wiz_main

Module code:

`timescale 1ns/1ps // 100 MHz -> 25 MHz (VGA pixel) and 24 MHz (camera XCLK). // Synthesis: MMCME2_BASE on Artix-7. Sim: behavioral clock generators. module clk_wiz_main ( input wire clk_100, output wire clk_25, output wire clk_24, output wire locked ); `ifdef SYNTHESIS // VCO = 100 * 12 / 1 = 1200 MHz (within 600..1600 MHz) // CLKOUT0 = 1200 / 48 = 25.000 MHz // CLKOUT1 = 1200 / 50 = 24.000 MHz wire clkfb; wire clk_25_unbuf, clk_24_unbuf; MMCME2_BASE #( .BANDWIDTH ("OPTIMIZED"), .CLKFBOUT_MULT_F (12.0), .CLKFBOUT_PHASE (0.0), .CLKIN1_PERIOD (10.0), .DIVCLK_DIVIDE (1), .CLKOUT0_DIVIDE_F (48.0), .CLKOUT0_PHASE (0.0), .CLKOUT1_DIVIDE (50), .CLKOUT1_PHASE (0.0), .STARTUP_WAIT ("FALSE") ) u_mmcm ( .CLKIN1 (clk_100), .CLKFBIN (clkfb), .CLKFBOUT (clkfb), .CLKOUT0 (clk_25_unbuf), .CLKOUT1 (clk_24_unbuf), .LOCKED (locked), .PWRDWN (1'b0), .RST (1'b0) ); BUFG u_buf25 (.I(clk_25_unbuf), .O(clk_25)); BUFG u_buf24 (.I(clk_24_unbuf), .O(clk_24)); `else reg clk_25_r = 1'b0; reg clk_24_r = 1'b0; always #20 clk_25_r = ~clk_25_r; // 40 ns period = 25 MHz always #21 clk_24_r = ~clk_24_r; // 42 ns period ≈ 23.8 MHz (close enough for sim) assign clk_25 = clk_25_r; assign clk_24 = clk_24_r; assign locked = 1'b1; `endif endmodule

Module Description:

Wraps an MMCME2_BASE primitive (synthesis) or behavioural clock generators (simulation). Produces clk_25 (VGA pixel) and clk_24 (camera XCLK) from a single 100 MHz board input. VCO = 100×12 / 1 = 1200 MHz; CLKOUT0 / 48 = 25 MHz, CLKOUT1 / 50 = 24 MHz. Outputs LOCKED for downstream reset.

## Module : cam_config

Module code:

`timescale 1ns/1ps // OV7670 power-up and register configuration sequencer. // Drives cam_rst_n, then walks a 77-entry SCCB-write ROM to configure RGB565 // output, color matrix, gamma curve, AGC/AEC, and the "magic" registers // (RSVD/CHLF/COM12/etc) the OV7670 needs for usable color. // // If test_pattern_en=1, two extra registers (SCALING_YSC + COM17) are written // after the ROM to enable the chip's internal color-bar test pattern. // // Status outputs (for LED diagnostics): // cfg_done - high once configuration finishes // reg_writes_done - count of completed ROM writes (0..79) // successful_xacts - count of writes where the slave ACK'd all three bytes module cam_config #( parameter integer RST_HOLD = 625000, // hold cam_rst_n low (25 ms @ 25 MHz) parameter integer POST_RST = 250000, // wait after release (10 ms) parameter integer SOFT_RST_WAIT = 750000, // wait after soft reset (30 ms; OV7670 datasheet min) parameter integer REG_GAP = 2500, // inter-register settling (~100 us) parameter [7:0] OV7670_ID = 8'h42 // SCCB write address ) ( input wire clk, input wire rst, output reg cam_rst_n, output reg cam_pwdn, output wire scl, inout wire sda, input wire test_pattern_en, output reg cfg_done, output reg [6:0] reg_writes_done, output reg [6:0] successful_xacts ); // ----------------------------------------------------------------- // SCCB master // ----------------------------------------------------------------- reg sccb_start; reg [7:0] sccb_id, sccb_sub, sccb_data; wire sccb_done, sccb_busy; wire sccb_ack_id, sccb_ack_sub, sccb_ack_data; sccb_master u_sccb ( .clk (clk), .rst (rst), .start (sccb_start), .id (sccb_id), .sub_addr (sccb_sub), .data (sccb_data), .done (sccb_done), .busy (sccb_busy), .ack_id (sccb_ack_id), .ack_sub (sccb_ack_sub), .ack_data (sccb_ack_data), .scl (scl), .sda (sda) ); // ----------------------------------------------------------------- // Register ROM (77 entries) — proven OV7670 init sequence. // {sub_addr[15:8], value[7:0]}; sentinel 16'hFFFF marks end. // ----------------------------------------------------------------- localparam integer NUM_REGS = 77; reg [6:0] rom_idx; reg [15:0] command; always @(*) begin case (rom_idx) // Soft reset (twice — OV7670 quirk) 7'h00: command = 16'h1280; 7'h01: command = 16'h1280; // Output format: RGB565 7'h02: command = 16'h1204; // COM7 RGB output 7'h03: command = 16'h1100; // CLKRC no prescale 7'h04: command = 16'h40D0; // COM15 RGB565 full range 7'h05: command = 16'h0C00; // COM3 no scaling 7'h06: command = 16'h3E00; // COM14 7'h07: command = 16'h0400; // COM1 disable CCIR656 7'h08: command = 16'h3A04; // TSLB 7'h09: command = 16'h1418; // COM9 AGC ceiling x4 // Color matrix 7'h0A: command = 16'h4FB3; 7'h0B: command = 16'h50B3; 7'h0C: command = 16'h5100; 7'h0D: command = 16'h523D; 7'h0E: command = 16'h53A7; 7'h0F: command = 16'h54E4; 7'h10: command = 16'h589E; 7'h11: command = 16'h3D80; // COM13 gamma enable // Window 7'h12: command = 16'h1711; 7'h13: command = 16'h1861; 7'h14: command = 16'h32A4; 7'h15: command = 16'h1903; 7'h16: command = 16'h1A7B; 7'h17: command = 16'h030A; 7'h18: command = 16'h0F4B; 7'h19: command = 16'h1E37; // "Magic" registers 7'h1A: command = 16'h330B; 7'h1B: command = 16'h3C78; 7'h1C: command = 16'h6900; 7'h1D: command = 16'h7400; 7'h1E: command = 16'hB084; // RSVD — required for good color 7'h1F: command = 16'hB10C; 7'h20: command = 16'hB20E; 7'h21: command = 16'hB380; // Scaling 7'h22: command = 16'h703A; 7'h23: command = 16'h7135; 7'h24: command = 16'h7211; 7'h25: command = 16'h73F0; 7'h26: command = 16'hA202; // Gamma curve 7'h27: command = 16'h7A20; 7'h28: command = 16'h7B10; 7'h29: command = 16'h7C1E; 7'h2A: command = 16'h7D35; 7'h2B: command = 16'h7E5A; 7'h2C: command = 16'h7F69; 7'h2D: command = 16'h8076; 7'h2E: command = 16'h8180; 7'h2F: command = 16'h8288; 7'h30: command = 16'h838F; 7'h31: command = 16'h8496; 7'h32: command = 16'h85A3; 7'h33: command = 16'h86AF; 7'h34: command = 16'h87C4; 7'h35: command = 16'h88D7; 7'h36: command = 16'h89E8; // AGC/AEC 7'h37: command = 16'h13E0; 7'h38: command = 16'h0000; 7'h39: command = 16'h1000; 7'h3A: command = 16'h0D40; 7'h3B: command = 16'h1418; 7'h3C: command = 16'hA505; 7'h3D: command = 16'hAB07; 7'h3E: command = 16'h2495; 7'h3F: command = 16'h2533; 7'h40: command = 16'h26E3; 7'h41: command = 16'h9F78; 7'h42: command = 16'hA068; 7'h43: command = 16'hA103; 7'h44: command = 16'hA6D8; 7'h45: command = 16'hA7D8; 7'h46: command = 16'hA8F0; 7'h47: command = 16'hA990; 7'h48: command = 16'hAA94; 7'h49: command = 16'h13E7; // COM8 — re-enable AGC + AEC + AWB // Misc 7'h4A: command = 16'h6900; 7'h4B: command = 16'h8C00; 7'h4C: command = 16'h6B4A; default: command = 16'hFFFF; endcase end // Color-bar extra writes (SCALING_YSC bit7 + COM17 DSP color bar) localparam integer NUM_CB_REGS = 2; reg [15:0] cb_rom [0:NUM_CB_REGS-1]; initial begin cb_rom[0] = 16'h7180; cb_rom[1] = 16'h4208; end // ----------------------------------------------------------------- // FSM // ----------------------------------------------------------------- localparam [3:0] S_RESET = 4'd0, S_POST_RST = 4'd1, S_SOFT_RST = 4'd2, S_WAIT_DONE = 4'd3, S_SOFT_WAIT = 4'd4, S_ROM_GO = 4'd5, S_ROM_WAIT = 4'd6, S_ROM_GAP = 4'd7, S_CB_GO = 4'd8, S_CB_WAIT = 4'd9, S_CB_GAP = 4'd10, S_DONE = 4'd11; reg [3:0] state; reg [19:0] timer; reg [4:0] cb_idx; initial begin state = S_RESET; timer = 20'd0; rom_idx = 7'd0; cb_idx = 5'd0; cam_rst_n = 1'b0; cam_pwdn = 1'b0; cfg_done = 1'b0; reg_writes_done = 7'd0; successful_xacts = 7'd0; sccb_start = 1'b0; sccb_id = 8'd0; sccb_sub = 8'd0; sccb_data = 8'd0; end always @(posedge clk) begin if (rst) begin state <= S_RESET; timer <= 20'd0; rom_idx <= 7'd0; cb_idx <= 5'd0; cam_rst_n <= 1'b0; cam_pwdn <= 1'b0; cfg_done <= 1'b0; reg_writes_done <= 7'd0; successful_xacts <= 7'd0; sccb_start <= 1'b0; end else begin sccb_start <= 1'b0; case (state) S_RESET: begin cam_rst_n <= 1'b0; cam_pwdn <= 1'b0; if (timer >= RST_HOLD - 1) begin cam_rst_n <= 1'b1; timer <= 20'd0; state <= S_POST_RST; end else timer <= timer + 20'd1; end S_POST_RST: begin if (timer >= POST_RST - 1) begin timer <= 20'd0; state <= S_SOFT_RST; end else timer <= timer + 20'd1; end S_SOFT_RST: begin if (!sccb_busy) begin sccb_start <= 1'b1; sccb_id <= OV7670_ID; sccb_sub <= 8'h12; // COM7 sccb_data <= 8'h80; // soft reset state <= S_WAIT_DONE; end end S_WAIT_DONE: begin if (sccb_done) begin if (!sccb_ack_id && !sccb_ack_sub && !sccb_ack_data) successful_xacts <= successful_xacts + 7'd1; timer <= 20'd0; state <= S_SOFT_WAIT; end end S_SOFT_WAIT: begin if (timer >= SOFT_RST_WAIT - 1) begin timer <= 20'd0; rom_idx <= 7'd0; state <= S_ROM_GO; end else timer <= timer + 20'd1; end S_ROM_GO: begin if (rom_idx >= NUM_REGS) begin cb_idx <= 5'd0; state <= test_pattern_en ? S_CB_GO : S_DONE; end else if (!sccb_busy) begin sccb_start <= 1'b1; sccb_id <= OV7670_ID; sccb_sub <= command[15:8]; sccb_data <= command[7:0]; state <= S_ROM_WAIT; end end S_ROM_WAIT: begin if (sccb_done) begin reg_writes_done <= reg_writes_done + 7'd1; if (!sccb_ack_id && !sccb_ack_sub && !sccb_ack_data) successful_xacts <= successful_xacts + 7'd1; timer <= 20'd0; state <= S_ROM_GAP; end end S_ROM_GAP: begin if (timer >= REG_GAP - 1) begin timer <= 20'd0; rom_idx <= rom_idx + 7'd1; state <= S_ROM_GO; end else timer <= timer + 20'd1; end S_CB_GO: begin if (cb_idx >= NUM_CB_REGS) begin state <= S_DONE; end else if (!sccb_busy) begin sccb_start <= 1'b1; sccb_id <= OV7670_ID; sccb_sub <= cb_rom[cb_idx][15:8]; sccb_data <= cb_rom[cb_idx][7:0]; state <= S_CB_WAIT; end end S_CB_WAIT: begin if (sccb_done) begin reg_writes_done <= reg_writes_done + 7'd1; if (!sccb_ack_id && !sccb_ack_sub && !sccb_ack_data) successful_xacts <= successful_xacts + 7'd1; timer <= 20'd0; state <= S_CB_GAP; end end S_CB_GAP: begin if (timer >= REG_GAP - 1) begin timer <= 20'd0; cb_idx <= cb_idx + 5'd1; state <= S_CB_GO; end else timer <= timer + 20'd1; end S_DONE: cfg_done <= 1'b1; default: state <= S_DONE; endcase end end endmodule

Module Description:

Boot-time SCCB sequencer. Drives cam_rst_n through a hold + release schedule, then walks a 77-entry register-write ROM (COM7 = RGB output, COM15 = RGB565 full range, plus AWB / gamma / scaling). Issues one 16-bit {reg, value} command at a time to sccb_master and waits for done before advancing. Also handles SW[8] camera color-bar test pattern.

## Module : sccb_master

Module code:

`timescale 1ns/1ps // SCCB 3-byte write master for OV7670. // Frame: START ¦ ID(8) ¦ DC ¦ SUB(8) ¦ DC ¦ DATA(8) ¦ DC ¦ STOP // // SCL period = 4 * HALF_PER_CYCLES clocks (two half-bit phases per bit, two for START/STOP). // Default HALF_PER_CYCLES = 128 -> ~49 kHz SCL at 25 MHz clk (well under SCCB max 400 kHz). // // Open-drain bus model: // scl, sda are 1 when released (rely on board-level pull-ups), 0 when driven low. // Master releases SDA during the 3 DC slots so the slave can ACK. // ACK is sampled at the midpoint of the SCL-high half on each DC slot. module sccb_master #( parameter integer HALF_PER_CYCLES = 128 // half SCL period in clk cycles ) ( input wire clk, input wire rst, input wire start, input wire [7:0] id, input wire [7:0] sub_addr, input wire [7:0] data, output reg done, output reg busy, output reg ack_id, output reg ack_sub, output reg ack_data, output wire scl, inout wire sda ); // Drive mask: 1 = master drives, 0 = master releases (DC / ACK slot) // Layout: [26:19] ID, [18] DC, [17:10] SUB, [9] DC, [8:1] DATA, [0] DC localparam [26:0] DRIVE_MASK = 27'b111111110_111111110_111111110; localparam [2:0] S_IDLE = 3'd0, S_START = 3'd1, // SCL=1, drop SDA -> START condition S_LOW = 3'd2, // SCL=0, set SDA to current bit S_HIGH = 3'd3, // SCL=1, hold SDA; sample ACK on DC slots S_STOP_A = 3'd4, // SCL=0, SDA=0 S_STOP_B = 3'd5, // SCL=1, SDA=0 S_STOP_C = 3'd6; // SCL=1, SDA released -> bus idle reg [2:0] state; reg [26:0] shift; reg [4:0] bit_idx; // 26 down to 0 reg [$clog2(HALF_PER_CYCLES)-1:0] div; reg scl_r; reg sda_low; // 1 = drive SDA low; 0 = release wire phase_done = (div == HALF_PER_CYCLES - 1); wire bit_drives = DRIVE_MASK[bit_idx]; wire drive_low_now = bit_drives && (shift[26] == 1'b0); // SDA: drive low if sda_low=1, else release. sda_in reads bus state. assign sda = sda_low ? 1'b0 : 1'bz; assign scl = scl_r ? 1'bz : 1'b0; wire sda_in = (sda === 1'bz) ? 1'b1 : sda; initial begin state = S_IDLE; shift = 27'd0; bit_idx = 5'd0; div = 0; scl_r = 1'b1; sda_low = 1'b0; busy = 1'b0; done = 1'b0; ack_id = 1'b1; ack_sub = 1'b1; ack_data = 1'b1; end always @(posedge clk) begin if (rst) begin state <= S_IDLE; div <= 0; scl_r <= 1'b1; sda_low <= 1'b0; busy <= 1'b0; done <= 1'b0; ack_id <= 1'b1; ack_sub <= 1'b1; ack_data <= 1'b1; end else begin done <= 1'b0; case (state) S_IDLE: begin scl_r <= 1'b1; sda_low <= 1'b0; if (start && !busy) begin // Pack with 1's in DC slots; mask suppresses driving anyway shift <= {id, 1'b1, sub_addr, 1'b1, data, 1'b1}; bit_idx <= 5'd26; busy <= 1'b1; sda_low <= 1'b1; // drop SDA while SCL high = START state <= S_START; div <= 0; end end S_START: begin scl_r <= 1'b1; sda_low <= 1'b1; if (phase_done) begin state <= S_LOW; div <= 0; end else div <= div + 1'b1; end S_LOW: begin scl_r <= 1'b0; sda_low <= drive_low_now; if (phase_done) begin state <= S_HIGH; div <= 0; end else div <= div + 1'b1; end S_HIGH: begin scl_r <= 1'b1; sda_low <= drive_low_now; // Sample ACK at midpoint of high phase on the 3 DC slots if (div == (HALF_PER_CYCLES >> 1) && !bit_drives) begin case (bit_idx) 5'd18: ack_id <= sda_in; 5'd9: ack_sub <= sda_in; 5'd0: ack_data <= sda_in; default: ; endcase end if (phase_done) begin div <= 0; if (bit_idx == 5'd0) begin state <= S_STOP_A; end else begin shift <= {shift[25:0], 1'b1}; bit_idx <= bit_idx - 5'd1; state <= S_LOW; end end else div <= div + 1'b1; end S_STOP_A: begin scl_r <= 1'b0; sda_low <= 1'b1; if (phase_done) begin state <= S_STOP_B; div <= 0; end else div <= div + 1'b1; end S_STOP_B: begin scl_r <= 1'b1; sda_low <= 1'b1; if (phase_done) begin state <= S_STOP_C; div <= 0; end else div <= div + 1'b1; end S_STOP_C: begin scl_r <= 1'b1; sda_low <= 1'b0; if (phase_done) begin state <= S_IDLE; busy <= 1'b0; done <= 1'b1; div <= 0; end else div <= div + 1'b1; end default: state <= S_IDLE; endcase end end endmodule

Module Description:

Bit-banged SCCB / 3-wire master. Generates START, byte transfers (device address 0x42 + W, register address, data) and STOP on SIOC / SIOD. The 9th bit of each byte is the SCCB don't-care slot (master releases SDA). ACK is sampled mid-SCL-high on each DC slot. Runs at ~49 kHz SCL (HALF_PER_CYCLES=128 at 25 MHz), well within OV7670 datasheet limit of 400 kHz.

## Module : cam_capture

Module code:

`timescale 1ns/1ps // OV7670 RGB565 capture -> YCbCr 4:2:2 quantization (Y3 + Cb2/Cr2). // // Pixel byte pairing: same proven technique as the 320 version. // - byte_sel toggles on every posedge PCLK while href is high // - byte_sel resets on href falling edge (prevents byte-swap drift) // - First byte stored in b1, second byte completes the RGB565 word // // Color math (BT.601 luma + signed-difference chroma): // Y8 = (77*R8 + 150*G8 + 29*B8) >> 8 // Cb2 = quantize(B8 - Y8) into 4 bins: 0(<-48), 1(-48..-1), 2(0..47), 3(>=48) // Cr2 = quantize(R8 - Y8) into 4 bins: same thresholds // // PIXEL_SKIP camera pixels at the start of each HREF line are discarded // (OV7670 HREF startup glitch). fb[0] per row = first valid camera pixel. // PIXEL_SKIP must be even to preserve chroma phase alignment. module cam_capture #( parameter IMG_WIDTH = 640, parameter IMG_HEIGHT = 480 ) ( input wire pclk_in, input wire [7:0] d_in, input wire href, input wire vsync, output reg [18:0] wr_addr, output reg [2:0] wr_luma, output reg [3:0] wr_chroma, output reg wr_en, output reg wr_chroma_en, output reg vsync_pulse ); // Suppress unused parameter warnings /* verilator lint_off UNUSED */ wire _unused = &{1'b0, IMG_WIDTH[9:1], IMG_HEIGHT[9:1]}; /* verilator lint_on UNUSED */ // Skip first N camera pixels per line (HREF startup glitch). Must be even. // Tune upward if black/garbage visible on right side of display. localparam [9:0] PIXEL_SKIP = 10'd16; localparam [9:0] VALID_COLS = 10'd640 - PIXEL_SKIP; // 624 // ----------------------------------------------------------------- // Byte pairing // ----------------------------------------------------------------- reg [7:0] b1; reg byte_sel; reg old_href; reg [9:0] pxl_cnt; reg [18:0] row_base; // base address of current fb row (0, 640, 1280, ...) reg [9:0] write_col; // column index within row (0..VALID_COLS-1) initial begin b1 = 8'd0; byte_sel = 1'b0; old_href = 1'b0; pxl_cnt = 10'd0; row_base = 19'd0; write_col = 10'd0; wr_addr = 19'd0; wr_luma = 3'd0; wr_chroma = 4'd0; wr_en = 1'b0; wr_chroma_en = 1'b0; vsync_pulse = 1'b0; end // ----------------------------------------------------------------- // RGB565 pixel word (assembled from two bytes) // ----------------------------------------------------------------- wire [15:0] pix16 = {b1, d_in}; // byte1=high, byte2=low wire [4:0] r5 = pix16[15:11]; wire [5:0] g6 = pix16[10:5]; wire [4:0] b5 = pix16[4:0]; wire [7:0] r8 = {r5, r5[4:2]}; wire [7:0] g8 = {g6, g6[5:4]}; wire [7:0] b8 = {b5, b5[4:2]}; // ----------------------------------------------------------------- // BT.601 luma: Y = (77R + 150G + 29B) / 256 // ----------------------------------------------------------------- wire [15:0] y_acc = ({8'd0, r8} * 16'd77) + ({8'd0, g8} * 16'd150) + ({8'd0, b8} * 16'd29); wire [7:0] y8 = y_acc[15:8]; wire [2:0] y3 = y8[7:5]; // ----------------------------------------------------------------- // Chroma: signed B-Y and R-Y quantized into 4 bins // ----------------------------------------------------------------- wire signed [9:0] y_s = $signed({2'b0, y8}); wire signed [9:0] b_s = $signed({2'b0, b8}); wire signed [9:0] r_s = $signed({2'b0, r8}); wire signed [9:0] diff_b = b_s - y_s; wire signed [9:0] diff_r = r_s - y_s; wire [1:0] cb2 = (diff_b >= 10'sd48) ? 2'd3 : (diff_b >= 10'sd0) ? 2'd2 : (diff_b >= -10'sd48) ? 2'd1 : 2'd0; wire [1:0] cr2 = (diff_r >= 10'sd48) ? 2'd3 : (diff_r >= 10'sd0) ? 2'd2 : (diff_r >= -10'sd48) ? 2'd1 : 2'd0; // ----------------------------------------------------------------- // Capture logic // ----------------------------------------------------------------- always @(posedge pclk_in) begin old_href <= href; vsync_pulse <= 1'b0; wr_en <= 1'b0; if (vsync) begin pxl_cnt <= 10'd0; byte_sel <= 1'b0; write_col <= 10'd0; row_base <= 19'd0; vsync_pulse <= 1'b1; end else if (href) begin if (byte_sel == 1'b0) begin b1 <= d_in; byte_sel <= 1'b1; end else begin // Write valid pixels: skip first PIXEL_SKIP, cap at VALID_COLS per row if (pxl_cnt >= PIXEL_SKIP && write_col < VALID_COLS) begin wr_en <= 1'b1; wr_luma <= y3; wr_chroma <= {cb2, cr2}; wr_chroma_en <= (write_col[0] == 1'b0); // chroma on even write_col wr_addr <= row_base + {9'd0, write_col}; write_col <= write_col + 10'd1; end pxl_cnt <= pxl_cnt + 10'd1; byte_sel <= 1'b0; end end else begin pxl_cnt <= 10'd0; byte_sel <= 1'b0; write_col <= 10'd0; if (old_href == 1'b1) row_base <= row_base + 19'd640; end end endmodule

Module Description:

Pixel ingest in cam_pclk domain. Captures RGB565 in two halves on consecutive HREF byte pulses, bit-replicates R5/G6/B5 to 8-bit channels, computes BT.601 luma Y8 = (77R + 150G + 29B) >> 8, truncates to Y3 = Y8[7:5], and quantizes diff_b = B-Y and diff_r = R-Y into 4 bins each (Cb2, Cr2). Writes Y per pixel and Cb/Cr only on even columns (4:2:2 subsampling). byte_sel resets on HREF fall to prevent byte-swap drift between lines. SW[9] injects a synthetic R/G/B vertical bar test pattern.

## Module : frame_buffer

Module code:

`timescale 1ns/1ps // Split luma + chroma frame buffer for YCbCr 4:2:2 storage. // // Luma: 307,200 entries x 3 bits (Y per pixel) ~30 BRAM36 tiles // Chroma: 153,600 entries x 4 bits (Cb2/Cr2 per pair) ~20 BRAM36 tiles // Total: ~50 / 50 BRAM36 on XC7A35T (full utilization). // // Chroma is shared between adjacent horizontal pixels (4:2:2 subsampling). // Even columns write chroma; odd columns read the same chroma word as the // preceding even column (rd_addr[18:1] selects the chroma pair). // // Read latency: 1 cycle (synchronous BRAM read). module frame_buffer ( input wire wr_clk, input wire wr_en, input wire [18:0] wr_addr, input wire [2:0] wr_luma, input wire [3:0] wr_chroma, input wire wr_chroma_en, input wire rd_clk, input wire [18:0] rd_addr, output reg [2:0] rd_luma, output reg [3:0] rd_chroma ); (* ram_style = "block" *) reg [2:0] luma_mem [0:307199]; (* ram_style = "block" *) reg [3:0] chroma_mem [0:153599]; wire [17:0] wr_chroma_addr = wr_addr[18:1]; wire [17:0] rd_chroma_addr = rd_addr[18:1]; always @(posedge wr_clk) begin if (wr_en) luma_mem[wr_addr] <= wr_luma; end always @(posedge wr_clk) begin if (wr_en && wr_chroma_en) chroma_mem[wr_chroma_addr] <= wr_chroma; end always @(posedge rd_clk) rd_luma <= luma_mem[rd_addr]; always @(posedge rd_clk) rd_chroma <= chroma_mem[rd_chroma_addr]; endmodule

Module Description:

Split dual-clock BRAM storage for YCbCr 4:2:2. luma_mem holds 307,200 entries × 3 bits; chroma_mem holds 153,600 entries × 4 bits (Cb2+Cr2 shared across each horizontal pair at address >> 1). Writes are clocked by cam_pclk; reads by clk_25. The dual-port BRAM primitives handle the clock-domain crossing at the byte level without a FIFO.

## Module : ycbcr_to_rgb444

Module code:

`timescale 1ns/1ps // YCbCr (Y3, Cb2, Cr2) -> RGB444. // // Bin layout (matches cam_capture thresholds): // bin 0 : diff < -48 -> strong deficit (e.g., very non-blue pixel for Cb) // bin 1 : diff -48..-1 -> mild deficit (slightly below neutral) // bin 2 : diff 0..47 -> mild positive (slightly above neutral) // bin 3 : diff >= 48 -> strong positive (e.g., strongly blue pixel for Cb) // // NEUTRAL rule: bins 1 and 2 straddle zero, so their offsets must be // antisymmetric: bin1 = -N, bin2 = +N for some small N. // This ensures a gray pixel (R=G=B) roundtrips back to R=G=B. // // With 2x2 Bayer dither on luma, y4 ranges 0..15. // R4 = clamp(y4 + r_off(Cr)) // B4 = clamp(y4 + b_off(Cb)) // G4 = clamp(y4 + g_off_cr(Cr) + g_off_cb(Cb)) module ycbcr_to_rgb444 ( input wire [2:0] in_y, input wire [1:0] in_cb, input wire [1:0] in_cr, input wire h_pos, // h_count[0] for dither input wire v_pos, // v_count[0] for dither output wire [11:0] out_rgb444 ); // Bayer 2x2 dither: alternates 0/1 in checkerboard pattern wire dither = h_pos ^ v_pos; wire [3:0] y4 = {in_y, dither}; // ----- Cr -> R offset ----- // Bin 1 (mild deficit) and bin 2 (mild positive) are antisymmetric reg signed [2:0] r_off; always @(*) case (in_cr) 2'd0: r_off = -3'sd3; // strong R deficit 2'd1: r_off = -3'sd1; // mild R deficit 2'd2: r_off = 3'sd1; // mild R positive default: r_off = 3'sd3; // strong R positive endcase // ----- Cb -> B offset ----- reg signed [2:0] b_off; always @(*) case (in_cb) 2'd0: b_off = -3'sd3; // strong B deficit 2'd1: b_off = -3'sd1; // mild B deficit 2'd2: b_off = 3'sd1; // mild B positive default: b_off = 3'sd3; // strong B positive endcase // ----- Cr -> G offset (approx -3/4 * r_off, antisymmetric) ----- reg signed [2:0] g_off_cr; always @(*) case (in_cr) 2'd0: g_off_cr = 3'sd2; // strong R deficit -> G boosted 2'd1: g_off_cr = 3'sd1; // mild R deficit -> G slightly boosted 2'd2: g_off_cr = -3'sd1; // mild R positive -> G slightly reduced default: g_off_cr = -3'sd2; // strong R positive -> G reduced endcase // ----- Cb -> G offset (approx -1/3 * b_off, antisymmetric) ----- reg signed [2:0] g_off_cb; always @(*) case (in_cb) 2'd0: g_off_cb = 3'sd1; // strong B deficit -> G boosted 2'd1: g_off_cb = 3'sd0; // mild B deficit -> no G change 2'd2: g_off_cb = 3'sd0; // mild B positive -> no G change default: g_off_cb = -3'sd1; // strong B positive -> G reduced endcase wire signed [4:0] g_off = g_off_cr + g_off_cb; // range: -4..+4 // ----- Sum and clamp ----- wire signed [5:0] r_sum = $signed({2'b0, y4}) + r_off; wire signed [5:0] b_sum = $signed({2'b0, y4}) + b_off; wire signed [5:0] g_sum = $signed({2'b0, y4}) + g_off; function [3:0] clamp4; input signed [5:0] x; begin if (x < 6'sd0) clamp4 = 4'd0; else if (x > 6'sd15) clamp4 = 4'd15; else clamp4 = x[3:0]; end endfunction assign out_rgb444 = {clamp4(r_sum), clamp4(g_sum), clamp4(b_sum)}; endmodule

Module Description:

Read-side decoder. Appends a Bayer 2×2 dither bit (h_pos XOR v_pos) to the 3-bit luma to get Y4 (range 0..15). Looks up antisymmetric offsets for R (from Cr), B (from Cb), and G (from both) using 4-entry case-statement LUTs. Sums offset + Y4, clamps to 0..15, and outputs 12-bit RGB444. Bin-1 (mild deficit) and bin-2 (mild positive) are antisymmetric so a neutral gray pixel round-trips correctly.

## Module : filter_pipeline

Module code:

`timescale 1ns/1ps // Filter selector / output pipeline. // // Mode (sw_mode): // 2'b00 RAW — YCbCr decoded to RGB444 (with dither) // 2'b01 INVERT — bitwise NOT of raw // 2'b10 COLOR_ISO — pass selected channel only (sw32: 00=R, 01=G, 10=B, 11=R) // 2'b11 EDGE — 3x3 Sobel on luma (threshold = sw74) // // Pipeline depth (frame buffer output -> rgb444_out): // raw / invert / iso : 1 cycle (latch raw_d) + 1 cycle (output reg) = 2 cycles // edge : 1 cycle (line_buffer3) + 1 cycle (output reg) = 2 cycles // Latencies are matched so mode switching doesn't introduce a horizontal shift. // // h_count / v_count are passed through for dither (raw) and edge gating (edge). module filter_pipeline #( parameter integer WIDTH = 640 ) ( input wire clk, input wire rst, input wire [2:0] fb_luma, input wire [3:0] fb_chroma, input wire [9:0] h_count, input wire [9:0] v_count, input wire pix_valid, input wire [1:0] sw_mode, input wire [1:0] sw32, input wire [3:0] sw74, input wire h_edge, input wire v_edge, output reg [11:0] rgb444_out ); // ----------------------------------------------------------------- // RAW: YCbCr -> RGB444 (combinational) // ----------------------------------------------------------------- wire [11:0] raw444; ycbcr_to_rgb444 u_ycbcr ( .in_y (fb_luma), .in_cb (fb_chroma[3:2]), .in_cr (fb_chroma[1:0]), .h_pos (h_count[0]), .v_pos (v_count[0]), .out_rgb444 (raw444) ); // INVERT and COLOR_ISO derived from raw444 wire [11:0] inv444 = ~raw444; wire [11:0] iso444 = (sw32 == 2'b00) ? {raw444[11:8], 4'h0, 4'h0 } : (sw32 == 2'b01) ? {4'h0, raw444[7:4], 4'h0 } : (sw32 == 2'b10) ? {4'h0, 4'h0, raw444[3:0]} : {raw444[11:8], 4'h0, 4'h0 }; // ----------------------------------------------------------------- // EDGE: 3x3 Sobel on luma (line_buffer3 has 1-cycle registered read, // filter_edge is combinational). // ----------------------------------------------------------------- wire [2:0] tl, tc, tr, ml, mc, mr, bl, bc, br; line_buffer3 #(.WIDTH(WIDTH)) u_lb3 ( .clk (clk), .rst (rst), .pix_in (fb_luma), .pix_valid (pix_valid), .h_count (h_count), .v_count (v_count), .top_l (tl), .top_c (tc), .top_r (tr), .mid_l (ml), .mid_c (mc), .mid_r (mr), .bot_l (bl), .bot_c (bc), .bot_r (br) ); wire [11:0] edge444; filter_edge u_edge ( .top_l (tl), .top_c (tc), .top_r (tr), .mid_l (ml), .mid_c (mc), .mid_r (mr), .bot_l (bl), .bot_c (bc), .bot_r (br), .threshold (sw74), .h_edge (h_edge), .v_edge (v_edge), .out_rgb444(edge444) ); // ----------------------------------------------------------------- // Latency-match register on non-edge paths (1 cycle, like line_buffer3) // ----------------------------------------------------------------- reg [11:0] raw_d, inv_d, iso_d; always @(posedge clk) begin raw_d <= raw444; inv_d <= inv444; iso_d <= iso444; end // ----------------------------------------------------------------- // Mode select + output register // ----------------------------------------------------------------- always @(posedge clk) begin if (rst) begin rgb444_out <= 12'h000; end else case (sw_mode) 2'b00: rgb444_out <= raw_d; 2'b01: rgb444_out <= inv_d; 2'b10: rgb444_out <= iso_d; default: rgb444_out <= edge444; endcase end endmodule

Module Description:

Read-side mode mux. SW[1:0] selects: 00=RAW (decoded YCbCr→RGB444), 01=INVERT (XOR 0xFFF), 10=COLOR_ISO (gate channels by SW[3:2]), 11=EDGE (Sobel on luma → white-on-black mask). All modes have equal 2-cycle latency so no pipeline mis-alignment occurs when switching modes. Instantiates ycbcr_to_rgb444, line_buffer3, and filter_edge.

## Module : line_buffer3

Module code:

`timescale 1ns/1ps // 3-line circular shift buffer for a 3x3 Sobel neighbourhood (luma only). // // Three WIDTH x 3-bit distributed RAMs hold rows N-2, N-1, N (current). // Spatial labels follow image convention: // top = row above center = row N-2 (oldest complete) // mid = center row = row N-1 (most recent complete) // bot = row below center = row N (currently being written) // // The Sobel pixel under test is (row N-1, col h-1). The 3x3 window samples // columns (h-2, h-1, h) of the three rows; col_r = h_count and col_l/c are // h_count clamped on subtraction. The window output appears 1 cycle after // the addresses are presented (registered read). // // HAZARD HANDLING: // Reading bot_r from lb[bot_idx][h_count] would return the value present // BEFORE this cycle's write (row N-3 leftover from the previous frame). To // get the just-arrived pixel we forward pix_in into bot_r directly, since // col_r is always equal to h_count in this design. bot_l and bot_c read // columns h-2 and h-1 which were written 2 and 1 cycles ago respectively // and are already committed. // // Display offset: the Sobel center at col h-1 emerges at VGA column h+1 // (after the filter_pipeline output register), giving a ~2-pixel rightward // and 1-row downward shift in edge mode. Invisible at 640x480. module line_buffer3 #( parameter integer WIDTH = 640 ) ( input wire clk, input wire rst, input wire [2:0] pix_in, input wire pix_valid, input wire [9:0] h_count, input wire [9:0] v_count, output reg [2:0] top_l, top_c, top_r, output reg [2:0] mid_l, mid_c, mid_r, output reg [2:0] bot_l, bot_c, bot_r ); /* verilator lint_off UNUSED */ wire _unused = &{1'b0, v_count}; /* verilator lint_on UNUSED */ (* ram_style = "distributed" *) reg [2:0] lb [0:2][0:WIDTH-1]; integer i, j; initial begin for (i = 0; i < 3; i = i + 1) for (j = 0; j < WIDTH; j = j + 1) lb[i][j] = 3'd0; end // wr_idx = which buffer is currently being written this row (= bot row). reg [1:0] wr_idx; initial wr_idx = 2'd0; wire [1:0] top_idx = (wr_idx == 2'd2) ? 2'd0 : wr_idx + 2'd1; // (wr+1) %3 wire [1:0] mid_idx = (wr_idx == 2'd0) ? 2'd2 : wr_idx - 2'd1; // (wr-1) %3 wire [1:0] bot_idx = wr_idx; // ----- Write current pixel into bot buffer ----- always @(posedge clk) begin if (pix_valid) lb[wr_idx][h_count] <= pix_in; end // ----- Advance wr_idx at end-of-line ----- always @(posedge clk) begin if (rst) wr_idx <= 2'd0; else if (pix_valid && h_count == WIDTH - 1) wr_idx <= (wr_idx == 2'd2) ? 2'd0 : wr_idx + 2'd1; end // ----- Window column indices (window spans h-2, h-1, h) ----- wire [9:0] col_l = (h_count >= 10'd2) ? h_count - 10'd2 : 10'd0; wire [9:0] col_c = (h_count >= 10'd1) ? h_count - 10'd1 : 10'd0; wire [9:0] col_r = h_count; // ----- Registered window outputs ----- // top, mid: read from completed buffers (no hazard). // bot: read cols h-2 and h-1 normally; forward pix_in into bot_r. always @(posedge clk) begin top_l <= lb[top_idx][col_l]; top_c <= lb[top_idx][col_c]; top_r <= lb[top_idx][col_r]; mid_l <= lb[mid_idx][col_l]; mid_c <= lb[mid_idx][col_c]; mid_r <= lb[mid_idx][col_r]; bot_l <= lb[bot_idx][col_l]; bot_c <= lb[bot_idx][col_c]; bot_r <= pix_in; // col_r == h_count, forward to avoid hazard end endmodule

Module Description:

Three-row sliding window for 2D filters. Shifts incoming Y3 samples through three distributed-RAM line stores so that filter_edge sees a 3×3 neighbourhood per cycle. The current pixel (bottom-right) is forwarded directly to avoid read-before-write on the bottom buffer.

## Module : filter_edge

Module code:

`timescale 1ns/1ps // 3x3 Sobel edge detector on Y (luma). // // Gx = (p02 + 2 p12 + p22) - (p00 + 2 p10 + p20) (right - left column sum) // Gy = (p20 + 2 p21 + p22) - (p00 + 2 p01 + p02) (bottom - top row sum) // mag = ¦Gx¦ + ¦Gy¦ (L1 approximation) // // Y is 3-bit (0..7). Each weighted column/row sum max = 7 + 14 + 7 = 28 -> 5 bits. // Gx, Gy fit in signed 6 bits; ¦Gx¦+¦Gy¦ fits in unsigned 7 bits. // // Output: white (12'hFFF) if mag > threshold AND not on a frame edge, // black (12'h000) otherwise. threshold is the SW[7:4] 4-bit value // zero-extended to the 7-bit comparator width. module filter_edge ( input wire [2:0] top_l, top_c, top_r, input wire [2:0] mid_l, mid_c, mid_r, input wire [2:0] bot_l, bot_c, bot_r, input wire [3:0] threshold, input wire h_edge, input wire v_edge, output wire [11:0] out_rgb444 ); wire signed [5:0] right_col = $signed({3'b0, top_r}) + $signed({2'b0, mid_r, 1'b0}) + $signed({3'b0, bot_r}); wire signed [5:0] left_col = $signed({3'b0, top_l}) + $signed({2'b0, mid_l, 1'b0}) + $signed({3'b0, bot_l}); wire signed [5:0] bot_row = $signed({3'b0, bot_l}) + $signed({2'b0, bot_c, 1'b0}) + $signed({3'b0, bot_r}); wire signed [5:0] top_row = $signed({3'b0, top_l}) + $signed({2'b0, top_c, 1'b0}) + $signed({3'b0, top_r}); wire signed [5:0] gx = right_col - left_col; wire signed [5:0] gy = bot_row - top_row; wire [5:0] abs_gx = gx[5] ? -gx : gx; wire [5:0] abs_gy = gy[5] ? -gy : gy; wire [6:0] mag = {1'b0, abs_gx} + {1'b0, abs_gy}; wire edge_on = (mag > {3'b0, threshold}); assign out_rgb444 = (h_edge ¦¦ v_edge) ? 12'h000 : edge_on ? 12'hFFF : 12'h000; endmodule

Module Description:

Sobel edge detector on 3-bit luma. Computes Gx (horizontal) and Gy (vertical) 3×3 gradient sums, approximates magnitude as |Gx| + |Gy| (L1 norm, avoids square-root). Thresholds against SW[7:4] to produce a binary edge mask. Frame-edge pixels are forced black to suppress artefacts from the out-of-bounds 3×3 window.

## Module : vga_timing

Module code:

`timescale 1ns/1ps // VGA 640x480 @ 60 Hz timing generator (25 MHz pixel clock). // // H: active=640, FP=16, sync=96, BP=48 -> total 800 // V: active=480, FP=10, sync=2, BP=33 -> total 525 // HSYNC and VSYNC are active-LOW. // // rd_addr is combinational. Pipeline: comb(0)+BRAM(1)+filter(2) = 3 cycles. // Pre-fetch starts 3 cycles early (last 3 H-blanking clocks) so pixel[0] // arrives exactly when active_d goes high at col 0. No right shift. // // line_base counts DOWN (479*640 -> 0) to correct upside-down camera output. module vga_timing ( input wire clk, input wire rst, output reg [9:0] h_count, output reg [9:0] v_count, output wire hsync, output wire vsync, output wire active_video, output wire fetch_active, output wire [18:0] rd_addr ); localparam H_ACTIVE = 10'd640; localparam H_FP = 10'd16; localparam H_SYNC = 10'd96; localparam H_BP = 10'd48; localparam H_TOTAL = H_ACTIVE + H_FP + H_SYNC + H_BP; // 800 localparam V_ACTIVE = 10'd480; localparam V_FP = 10'd10; localparam V_SYNC = 10'd2; localparam V_BP = 10'd33; localparam V_TOTAL = V_ACTIVE + V_FP + V_SYNC + V_BP; // 525 localparam H_SYNC_START = H_ACTIVE + H_FP; // 656 localparam H_SYNC_END = H_ACTIVE + H_FP + H_SYNC; // 752 localparam V_SYNC_START = V_ACTIVE + V_FP; // 490 localparam V_SYNC_END = V_ACTIVE + V_FP + V_SYNC; // 492 // Pipeline pre-fetch depth localparam PIPE = 10'd3; initial begin h_count = 10'd0; v_count = 10'd0; end // ----------------------------------------------------------------- // Pixel counters // ----------------------------------------------------------------- always @(posedge clk) begin if (rst) begin h_count <= 10'd0; v_count <= 10'd0; end else if (h_count == H_TOTAL - 10'd1) begin h_count <= 10'd0; v_count <= (v_count == V_TOTAL - 10'd1) ? 10'd0 : v_count + 10'd1; end else begin h_count <= h_count + 10'd1; end end assign hsync = ~((h_count >= H_SYNC_START) && (h_count < H_SYNC_END)); assign vsync = ~((v_count >= V_SYNC_START) && (v_count < V_SYNC_END)); assign active_video = (h_count < H_ACTIVE) && (v_count < V_ACTIVE); // ----------------------------------------------------------------- // line_base counts DOWN to flip image vertically (fix upside-down). // Start: (479-V_OFFSET)*640 — skip V_OFFSET blank rows from camera // Each line: subtract 640 (go one row up in buffer) // Frame end: reset to starting row // ----------------------------------------------------------------- localparam [18:0] LINE_TOP = 19'd479 * 19'd640; reg [18:0] line_base; initial line_base = LINE_TOP; always @(posedge clk) begin if (rst) begin line_base <= LINE_TOP; end else if (h_count == H_TOTAL - 10'd1) begin if (v_count == V_TOTAL - 10'd1) line_base <= LINE_TOP; else if (v_count < V_ACTIVE) // all 480 active lines line_base <= line_base - 19'd640; end end // ----------------------------------------------------------------- // rd_addr: mirrored + 3-cycle pre-fetch to eliminate structural black // pixels at left from BRAM(1)+filter(2)=3 pipeline depth. // Pre-fetch at h_count=797..799 uses next line's base (line_base-640), // filling display pixels 0..2 so active_d can rise at h_count=0. // fetch_active = active_video ¦ prefetch; top.v pipes it through // active_pipe so active_d = active_pipe[2] & active_video is valid // from h_count=0 on all lines except row 0 and row 479 (frame edges). // Coefficient 998 = floor(623*1024/639). Must match PIXEL_SKIP in cam_capture.v. // ----------------------------------------------------------------- localparam [9:0] PIXEL_SKIP = 10'd16; localparam [9:0] VALID_COLS = 10'd640 - PIXEL_SKIP; // 624 localparam [9:0] PREFETCH_START = 10'd797; // H_TOTAL - PIPE wire in_active = (h_count < H_ACTIVE) && (v_count < V_ACTIVE); wire prefetch = (h_count >= PREFETCH_START) && (v_count < V_ACTIVE - 10'd1); wire [9:0] pre_h = h_count - PREFETCH_START; wire [9:0] act_h = h_count + PIPE; wire [9:0] eff_h = prefetch ? pre_h : act_h; wire [18:0] next_line_base = line_base - 19'd640; wire [18:0] eff_base = prefetch ? next_line_base : line_base; wire [19:0] h_scaled = ({10'd0, eff_h} * 20'd998) >> 10; wire [9:0] mirror_x = (VALID_COLS - 10'd1) - h_scaled[9:0]; assign fetch_active = in_active ¦¦ prefetch; assign rd_addr = fetch_active ? (eff_base + {9'd0, mirror_x}) : 19'd0; endmodule

Module Description:

VESA 640×480 @ 60 Hz timing generator. H_TOTAL=800 (active=640, FP=16, sync=96, BP=48); V_TOTAL=525 (active=480, FP=10, sync=2, BP=33). Produces hsync/vsync (active-low), active_video, and fetch_active (extends 3 cycles past active region for pre-fetch). rd_addr is computed combinationally with vertical flip (line_base counts down from row 479→0) and horizontal mirror (h_scaled = eff_h × 998 >> 10, mirror_x = VALID_COLS-1 − h_scaled).

## Module : debouncer

Module code:

`timescale 1ns/1ps // W-bit debouncer with two-flop synchronizer + N-bit stability counter. // Output bit transitions only after the synchronized input is stable for 2^N clocks. module debouncer #( parameter W = 16, parameter N = 20 ) ( input wire clk, input wire [W-1:0] sw_in, output reg [W-1:0] sw_out ); initial sw_out = {W{1'b0}}; genvar i; generate for (i = 0; i < W; i = i + 1) begin : g_bit reg [N-1:0] cnt; reg s0, s1; initial begin cnt = {N{1'b0}}; s0 = 1'b0; s1 = 1'b0; end always @(posedge clk) begin s0 <= sw_in[i]; s1 <= s0; if (s1 == sw_out[i]) begin cnt <= {N{1'b0}}; end else if (&cnt) begin sw_out[i] <= s1; cnt <= {N{1'b0}}; end else begin cnt <= cnt + 1'b1; end end end endgenerate endmodule

Module Description:

Generic W-bit counter-based debouncer with a per-bit two-flop synchronizer. With N=20 stability counter, an input must be stable for 2²⁰ clocks (~42 ms at 25 MHz) before the output flips. Used for all 16 switches and BTNC.

## Module : cdc_pulse_sync

Module code:

`timescale 1ns/1ps // Toggle-based pulse CDC: 1-cycle pulse in clk_src -> 1-cycle pulse in clk_dst. // Requires pulse_in spacing >= 2 clk_dst periods. module cdc_pulse_sync ( input wire clk_src, input wire clk_dst, input wire rst_src, input wire rst_dst, input wire pulse_in, output wire pulse_out ); reg toggle_src; initial toggle_src = 1'b0; always @(posedge clk_src) begin if (rst_src) toggle_src <= 1'b0; else if (pulse_in) toggle_src <= ~toggle_src; end (* ASYNC_REG = "TRUE" *) reg s1, s2, s3; always @(posedge clk_dst) begin if (rst_dst) {s1, s2, s3} <= 3'b0; else {s1, s2, s3} <= {toggle_src, s1, s2}; end assign pulse_out = s2 ^ s3; endmodule

Module Description:

Single-pulse CDC synchronizer. Toggle-flop in the source domain (cam_pclk), three-FF synchronizer + edge-detect in the destination domain (clk_25). Flops are marked (* ASYNC_REG = "TRUE" *) so Vivado places them in adjacent flip-flops for minimum metastability MTBF. Used to bring the camera VSYNC pulse into clk_25 for the frame-rate LED toggle.

# Simulation & Testbenches

All major modules have dedicated cocotb 2.x Python testbenches under tests/640/ (for the rtl/ path). Testbenches compile and simulate with Icarus Verilog via python run_tests.py.

How to run all testbenches:

cd tests/640 python run_tests.py # all six targets python run_tests.py vga_timing filter_pipeline # subset

## Testbench : test_vga_timing

Module under test + what is verified:

Verifies the VESA 640×480 @60 Hz timing generator (rtl/vga_timing.v). Drives a 25 MHz clock and runs for >1 full frame (>420,000 cycles). Checks h_count wraps 0..799, v_count wraps 0..524 (V_TOTAL=525), hsync LOW for h in [656..751], vsync LOW for v in [490..491], active_video HIGH iff h<640 and v<480, and rd_addr=0 outside active region.

Specific checks:

h_count wraps 0..799; max observed = 799.

v_count wraps 0..524 (V_TOTAL=525); max observed = 524.

hsync is LOW exactly when h ∈ [656..751], HIGH otherwise.

vsync is LOW exactly when v ∈ [490..491], HIGH otherwise.

fetch_active covers active region plus 3-cycle prefetch window.

rd_addr forced to 0 when !fetch_active.

Testbench source: tests/640/test_vga_timing.py

Simulation result (python run_tests.py vga_timing):

** TESTS=4 PASS=4 FAIL=0 SKIP=0 **

## Testbench : test_sccb_master

Module under test + what is verified:

Verifies the bit-banged SCCB master (rtl/sccb_master.v). Compiled with HALF_PER_CYCLES=4 to keep simulation to ~250 cycles per transaction instead of ~7900. Open-drain bus with no slave attached so all three ACK slots read back 1 (NAK).

Specific checks:

Idle state: busy=0, done=0 after reset.

start pulse → busy rises; done pulses exactly once; busy falls next cycle.

done stays high for exactly one clock cycle.

Second start during busy is ignored — only one done pulse results.

rst mid-transaction immediately clears busy and done.

ack_id/ack_sub/ack_data = 1 with no slave on the bus.

Testbench source: tests/640/test_sccb_master.py

Simulation result (python run_tests.py sccb_master):

** TESTS=6 PASS=6 FAIL=0 SKIP=0 **

## Testbench : test_frame_buffer

Module under test + what is verified:

Verifies the dual-clock split frame buffer (rtl/frame_buffer.v) storing Y3 per pixel and Cb2/Cr2 per pixel pair (YCbCr 4:2:2).

Specific checks:

Luma write+readback correct at addresses 0, 1, 2, 3, 1000.

Chroma write at even addr 0 is shared with odd addr 1 (same chroma word).

wr_chroma_en=0 on odd columns does not corrupt the shared chroma word.

BRAM 1-cycle read latency respected in all checks.

Testbench source: tests/640/test_frame_buffer.py

Simulation result (python run_tests.py frame_buffer):

** TESTS=3 PASS=3 FAIL=0 SKIP=0 **

## Testbench : test_filter_pipeline

Module under test + what is verified:

Verifies the 4-mode filter mux (rtl/filter_pipeline.v), which instantiates ycbcr_to_rgb444, line_buffer3, and filter_edge.

Specific checks:

RAW mode: rgb444_out is non-zero for mid-gray luma + neutral chroma.

INVERT mode: rgb444_out == ~raw_sample & 0xFFF.

COLOR_ISO R: only the R nibble is non-zero.

COLOR_ISO G: only the G nibble is non-zero.

COLOR_ISO B: only the B nibble is non-zero.

EDGE on flat field (luma=4 across 3×640): rgb444_out == 0x000 (no gradient).

Testbench source: tests/640/test_filter_pipeline.py

Simulation result (python run_tests.py filter_pipeline):

** TESTS=6 PASS=6 FAIL=0 SKIP=0 **

## Testbench : test_cam_capture

Module under test + what is verified:

Verifies the OV7670 pixel ingest (rtl/cam_capture.v).

Specific checks:

First PIXEL_SKIP=16 pixels per HREF → wr_en=0 (OV7670 startup glitch).

Pure-red RGB565 (0xF800) → wr_luma=2, wr_chroma non-zero.

vsync pulse asserts vsync_pulse for one cycle, clears wr_en.

After second HREF line, wr_addr ≥ 640 (row_base advanced).

wr_chroma_en alternates 1,0,1,0... (4:2:2 even-column write).

Testbench source: tests/640/test_cam_capture.py

Simulation result (python run_tests.py cam_capture):

** TESTS=5 PASS=5 FAIL=0 SKIP=0 **

## Testbench : test_ycbcr_to_rgb444

Module under test + what is verified:

Verifies the combinational YCbCr decoder (rtl/ycbcr_to_rgb444.v). No clock needed; drive inputs, await 1 ns propagation, check output.

Specific checks:

Six known (in_y, in_cb, in_cr, h_pos, v_pos) → out_rgb444 match reference model.

Large positive offset clamps to 15, not wrap (y4=14 + r_off=+3 → 15).

Large negative offset clamps to 0, not underflow (y4=0 + r_off=-3 → 0).

Flipping h_pos changes dither, shifts y4 by 1, changes all channels.

All 32 (in_cb, in_cr, h_pos) combinations at mid-luma produce defined output.

Testbench source: tests/640/test_ycbcr_to_rgb444.py

Simulation result (python run_tests.py ycbcr_to_rgb444):

** TESTS=5 PASS=5 FAIL=0 SKIP=0 **

## cocotb Testbenches — 320-path (sources_1/new/)

Six Python testbenches under tests/320/ cover the baseline 320×240 pipeline. Run with: cd tests/320 && python run_tests.py

## Testbench : test_camera_capture

Module under test + what is verified:

Verifies camera_capture_640x320 (sources_1/new/camera_capture_640x320.v). Drives PCLK / HREF / VSYNC and checks byte-pair assembly, pixel-skip, line_cnt even/odd gating, and byte_sel reset on HREF fall.

Specific checks:

First PIXEL_SKIP bytes per HREF: write_en=0.

Full pixel pair on even line produces write_en=1 with correct RGB444.

byte_sel resets to 0 on HREF falling edge (prevents byte-swap drift).

Odd line_cnt (1,3,...): write_en=0 — only even lines are kept.

Even line after odd: write_en=1 (downsampler resumes).

Testbench source: tests/320/test_camera_capture.py

Simulation result (python run_tests.py camera_capture):

** TESTS=5 PASS=5 FAIL=0 SKIP=0 **

## Testbench : test_vga_display

Module under test + what is verified:

Verifies vga_640x320_display (sources_1/new/vga_640x320_display.v). Checks VGA counter bounds, sync polarity, and horizontal upscale formula.

Specific checks:

h_cnt wraps 0..799 (HTOTAL=800).

v_cnt wraps 0..524 (VTOTAL=525).

hsync LOW in h=[656..751]; vsync LOW in v=[490..491].

active_d HIGH iff h<640 and v<480.

frame_addr upscale: img_x=(h_cnt*496)>>10, img_y=v_cnt>>1.

Testbench source: tests/320/test_vga_display.py

Simulation result (python run_tests.py vga_display):

** TESTS=5 PASS=5 FAIL=0 SKIP=0 **

## Testbench : test_frame_buffer

Module under test + what is verified:

Verifies frame_buffer_640x320 (sources_1/new/frame_buffer_640x320.v). 76,800 × 12-bit dual-clock BRAM: write @ pclk, read @ 25 MHz.

Specific checks:

Write+readback correct at address 0, 1, 5, 100.

BRAM 1-cycle read latency respected in all checks.

Testbench source: tests/320/test_frame_buffer.py

Simulation result (python run_tests.py frame_buffer):

** TESTS=3 PASS=3 FAIL=0 SKIP=0 **

## Testbench : test_sccb_config

Module under test + what is verified:

Verifies sccb_config (sources_1/new/sccb_config.v). 8-register minimal OV7670 init sequence over a bit-banged SCCB bus.

Specific checks:

Idle: busy=0 before start.

start pulse → busy rises within 1 cycle.

done pulses once per 3-byte transaction.

All 8 register writes complete; done_all asserts after last write.

Testbench source: tests/320/test_sccb_config.py

Simulation result (python run_tests.py sccb_config):

** TESTS=4 PASS=4 FAIL=0 SKIP=0 **

## Testbench : test_line_buffer3

Module under test + what is verified:

Verifies line_buffer3 (sources_1/new/line_buffer3.v). Three-row distributed-RAM sliding window; WIDTH=8 for fast simulation.

Specific checks:

After 3 full lines loaded: all 9 window positions hold expected values.

Top-left corner of window updates correctly on each new pixel.

Middle row values persist while bottom row advances.

Forward path: bot_r equals current pix_in (no read-before-write stale).

Wrap: window shifts correctly across a full line boundary.

Testbench source: tests/320/test_line_buffer3.py

Simulation result (python run_tests.py line_buffer3):

** TESTS=5 PASS=5 FAIL=0 SKIP=0 **

## Testbench : test_filter_edge

Module under test + what is verified:

Verifies filter_edge (sources_1/new/filter_edge.v). 3×3 Sobel on 3-bit luma with L1 magnitude and threshold from sw_threshold.

Specific checks:

Flat field (all 9 pixels identical): output=0 (no gradient).

Vertical step edge: Gx large, output > threshold → edge detected.

Horizontal step edge: Gy large, output > threshold → edge detected.

Diagonal: both Gx and Gy contribute.

Frame-edge pixels (h_edge or v_edge): output forced to 0.

Threshold sweep: lower threshold finds more edges, higher threshold fewer.

Strong edge with threshold=0: output always non-zero.

Testbench source: tests/320/test_filter_edge.py

Simulation result (python run_tests.py filter_edge):

** TESTS=7 PASS=7 FAIL=0 SKIP=0 **

## Legacy Verilog Testbenches (sim/)

Pre-cocotb Verilog testbenches compiled and run directly with iverilog + vvp. All pass under iverilog -g2012. VCD waveforms viewable in GTKWave.

How to run:

# from repo root iverilog -g2012 -o sim/tb_vga_timing.vvp sim/tb_vga_timing.v rtl/vga_timing.v iverilog -g2012 -o sim/tb_sccb_master.vvp sim/tb_sccb_master.v rtl/sccb_master.v iverilog -g2012 -o sim/tb_frame_buffer.vvp sim/tb_frame_buffer.v rtl/frame_buffer.v iverilog -g2012 -o sim/tb_filter_pipeline.vvp sim/tb_filter_pipeline.v \ rtl/filter_pipeline.v rtl/ycbcr_to_rgb444.v rtl/line_buffer3.v rtl/filter_edge.v for tb in tb_vga_timing tb_sccb_master tb_frame_buffer tb_filter_pipeline; do vvp sim/$tb.vvp done

## Testbench : tb_vga_timing

Module under test + what is verified:

Verifies VGA 640x480 @60 Hz timing (rtl/vga_timing.v) over one full frame (800x525 = 420,000 cycles). Self-checking: asserts hsync LOW in h=[656..751], vsync LOW in v=[490..491], active_video HIGH iff h<640 && v<480, rd_addr=0 outside active region.

Specific checks:

h_count wraps 0..799 (max observed = 799).

v_count wraps 0..524 (V_TOTAL=525; max observed = 524).

hsync is LOW exactly when h in [656..751], HIGH otherwise.

vsync is LOW exactly when v in [490..491], HIGH otherwise.

active_video HIGH iff h<640 and v<480.

rd_addr forced to 19'd0 outside active region.

Testbench source: sim/tb_vga_timing.v

Simulation result (iverilog -g2012 + vvp):

PASS: VGA timing OK (max_h=799 max_v=524)

## Testbench : tb_sccb_master

Module under test + what is verified:

Verifies the bit-banged SCCB master (rtl/sccb_master.v), compiled with HALF_PER_CYCLES=4 for fast simulation. Pull-ups on SDA/SCL emulate an idle bus (no slave). All three ACK slots read back 1 (NAK).

Specific checks:

START condition detected: SDA falls while SCL high.

At least 27 SCL rising edges (one per bit slot).

STOP condition detected: SDA rises while SCL high.

done pulses exactly once per transaction.

busy clears on the cycle after done.

ack_id, ack_sub, ack_data = 1 (no slave attached).

Testbench source: sim/tb_sccb_master.v

Simulation result (iverilog -g2012 + vvp):

PASS: SCCB master: START+27 bits+STOP, done=1, scl_rises=28

## Testbench : tb_frame_buffer

Module under test + what is verified:

Verifies the dual-clock split BRAM (rtl/frame_buffer.v). Writes known luma and chroma values to addresses 0/1/2/3/1000 and reads them back, checking both luma round-trip and 4:2:2 chroma sharing across even/odd pairs.

Specific checks:

Luma writes at addr 0,1,2,3,1000 read back the exact value.

Chroma write at even addr 0 is shared with odd addr 1 (same chroma word).

wr_chroma_en=0 on odd column does not corrupt the shared chroma word.

BRAM 1-cycle read latency respected.

Testbench source: sim/tb_frame_buffer.v

Simulation result (iverilog -g2012 + vvp):

PASS: frame_buffer luma + 4:2:2 chroma sharing OK

## Testbench : tb_filter_pipeline

Module under test + what is verified:

Verifies the 4-mode filter mux (rtl/filter_pipeline.v). Drives constant gray luma + neutral chroma, cycles sw_mode through all 4 codes, and checks registered output. EDGE on flat field must yield 12'h000.

Specific checks:

RAW mode: rgb444_out is non-zero (mid-gray decode).

INVERT mode: rgb444_out == ~raw_sample.

COLOR_ISO R: only the R nibble is non-zero.

COLOR_ISO G: only the G nibble is non-zero.

COLOR_ISO B: only the B nibble is non-zero.

EDGE on flat field (constant luma): rgb444_out == 12'h000.

Testbench source: sim/tb_filter_pipeline.v

Simulation result (iverilog -g2012 + vvp):

PASS: filter_pipeline 4 modes OK (raw=979)

# Challenge Faced

OV7670 finicky power-up timing — Reset hold, post-reset wait, and per-register SCCB gap all need to be tuned precisely, or the sensor either ignores writes or boots into a wrong output format (e.g. YUV instead of RGB565).

BRAM is the hard wall on XC7A35T — Storing the full 640×480 frame in RGB565 does not fit (4.9 Mb > 1.8 Mb on-chip). The design had to commit to YCbCr 4:2:2 with aggressive bit-depth quantization (Y3, Cb2, Cr2 = 5 bits/pixel) before any feature work could land.

Two clock domains + non-clock-capable PCLK pin — cam_pclk arrives on A16 (not a clock-capable input on Basys 3), so CLOCK_DEDICATED_ROUTE FALSE is required in the XDC. CDC crossings are confined to dual-port BRAM and a dedicated cdc_pulse_sync rather than letting them appear ad-hoc across the design.

Color reproduction tuning — Early versions had a green bias caused by rounding inside the chroma decoder and by bin thresholds that were not antisymmetric. Fixing required choosing thresholds (-48, 0, +48) and LUT offset tables that round-trip a neutral gray pixel correctly.

VGA pipeline alignment — Getting the 3-cycle latency (BRAM + filter_pipeline) to align exactly with active_video required a 3-cycle shift register (active_pipe) and a 3-cycle pre-fetch window (h_count 797-799) in vga_timing. Earlier builds had a 4-pixel rightward content shift from an over-pipelined rd_addr register.

Sobel on 3-bit luma is coarse — With only 8 luma levels, threshold tuning matters more than usual. The Bayer dither on the decode side (effectively Y4 instead of Y3 to the filter) recovered significant detail at the cost of a checkerboard artefact on smooth gradients.

# AI Usage Declaration

Per the project rubric (pass/fail requirement), this section discloses any use of AI tools during the project.

Tools used:

Anthropic Claude (Claude Code CLI) — used to draft and iteratively refine the Verilog modules (cam_capture, ycbcr_to_rgb444, filter_pipeline, line_buffer3, filter_edge, vga_timing, sccb_master, frame_buffer, top.v), the cocotb 2.x Python testbenches in tests/640/ (test_cam_capture, test_vga_timing, test_frame_buffer, test_filter_pipeline, test_sccb_master, test_ycbcr_to_rgb444), and the legacy Verilog testbenches in sim/. All generated code was inspected, edited, and validated on hardware (Basys 3 + OV7670) and in simulation (Icarus Verilog) by the team.

Anthropic Claude — used to help structure and draft this report (block diagram ASCII art, module description blocks, Q&A entries in the Design Decision section, challenge bullet points). Final wording and decisions reflect the team's actual experience.

Specific applications:

Code generation: SCCB FSM scaffolding, BT.601 luma multiply, antisymmetric chroma quantization bins, 3x3 line buffer sliding window, VGA 640x480 timing constants, cocotb test coroutines (ClockCycles, Timer, RisingEdge helpers).

Debugging assistance: explaining iverilog scheduling races, narrowing the green-bias bug in the chroma decoder (non-antisymmetric bin offsets), locating the byte-pairing reset point in cam_capture, fixing V_TOTAL=521→525 (V_BP=29→33) to eliminate bottom black strip.

Documentation: drafting this document and the README.

Not done by AI:

Hardware bring-up, oscilloscope / scope-on-LED debug, pin mapping verification, on-board demonstration tuning, and final architectural decisions (YCbCr 4:2:2 storage, Y3+Cb2+Cr2 bit depth, dither scheme, threshold values, V_TOTAL fix root-cause diagnosis).